Ghana - Settlement-to-School Walking Distance Map

Access to education is foundational to human development, yet across Ghana, a substantial share of settlements remains situated well beyond the practical walking distance of any school. You are required to perform a spatial analysis that identifies those settlements by comparing the locations of education facilities against the distribution of populated places across the study area. Settlements falling outside a 5-kilometre proximity threshold from any school are flagged as underserved, producing a spatially accurate, decision-ready dataset that education planners, district assemblies, and development partners can use to prioritise where new facilities, feeder roads, or school infrastructure investments are most urgently needed.

You will be working with three layers: education_facilities.gpkg (point layer of schools), settlements.gpkg (point layer of populated places), and roads.gpkg (line layer of the road network, included solely to provide transport context for the map). The project and all three layers must be in EPSG:25000 - Leigon / Ghana Metre Grid, before any processing begins, as all distance thresholds in this task are defined in metric units and depend on that CRS being active.

The core of the analysis is determining which settlements fall outside the effective proximity reach of any school. Begin by generating a single dissolved polygon buffer at a fixed radius of 5,000 metres around all school locations, saved as school_5km_buffer.gpkg. This unified corridor represents the on-threshold service zone and must not be altered. Using the reprojected school layer as the distance reference, each settlement must then be evaluated for its distance to the nearest school using a nearest-feature spatial join. The result is settlements_school_tagged.gpkg: a point layer that retains every feature from the input settlements layer with a new text field named school_access, populated with Within 5km for settlements at or below the 5,000-metre threshold and Beyond 5km for those that exceed it. Total feature count must be identical to the input settlements layer. This tagged layer is the primary analytical deliverable.

From settlements_school_tagged.gpkg, produce settlements_beyond_5km.gpkg as an attribute-filtered subset containing only features where school_access = 'Beyond 5km'. In the QGIS project, settlements_school_tagged.gpkg must be symbolised using school_access as the classification field: blue circles for Within 5km settlements and red circles for Beyond 5km settlements. The education facilities layer must be displayed as green diamonds, roads as thin grey lines, and the buffer layer as a semi-transparent yellow fill. All layers must be loaded into the project.

Deliver a project folder named Settlement-to-school walking distance map containing all input and derived output layers alongside the project file, Settlement_school_walking_distance.qgz saved with relative paths. The project must reopen without broken layer links, CRS mismatches, or missing files.